# Variables information

This page describes configuration variables and their default values.

## Required variables

| Variable | Description |
| --- | --- |
| DESIGN_NAME | The name of the top level module of the design |
| VERILOG_FILES | The path of the design's verilog files |
| CLOCK_PERIOD | The clock period for the design in ns |
| CLOCK_NET | The name of the Net input to root clock buffer used in Clock Tree Synthesis. |
| CLOCK_PORT | The name of the design's clock port used in Static Timing Analysis. |

## Optional variables

These variables are optional that can be specified in the design configuration file.

### Synthesis

| Variable | Description |
| --- | --- |
| LIB_SYNTH | The library used for synthesis by yosys. (Default: $::env(PDK_ROOT)/$::env(PDK)/libs.ref/$::env(STD_CELL_LIBRARY)/lib/sky130_fd_sc_hd__tt_025C_1v80.lib) |
| SYNTH_BIN | The yosys binary used in the flow. (Default: yosys) |
| SYNTH_DRIVING_CELL | The cell to drive the input ports. (Default: sky130_fd_sc_hd__inv_8) |
| SYNTH_DRIVING_CELL_PIN | The name of the SYNTH_DRIVING_CELL output pin. (Default: Y) |
| SYNTH_CAP_LOAD | The capacitive load on the output ports in femtofarads. (Default: 17.65 ff) |
| SYNTH_MAX_FANOUT | The max load that the output ports can drive. (Default: 5 cells) |
| SYNTH_MAX_TRAN | The max transition time (slew) from high to low or low to high on cell inputs in ns. Used in synthesis (Default: Calculated at runtime as 10% of the provided clock period, unless this exceeds a set DEFAULT_MAX_TRAN, in which case it will be used as is). |
| SYNTH_STRATEGY | Strategies for abc logic synthesis and technology mapping Possible values are DELAY/AREA 0-3/0-2; the first part refers to the optimization target of the synthesis strategy (area vs. delay) and the second one is an index. (Default: AREA 0) |
| SYNTH_BUFFERING | Enables abc cell buffering Enabled = 1, Disabled = 0 (Default: 1) |
| SYNTH_SIZING | Enables abc cell sizing (instead of buffering) Enabled = 1, Disabled = 0 (Default: 0) |
| SYNTH_READ_BLACKBOX_LIB | A flag that enable reading the full(untrimmed) liberty file as a blackbox for synthesis. Please note that this is not used in technology mapping. This should only be used when trying to preserve gate instances in the rtl of the design. Enabled = 1, Disabled = 0 (Default: 0) |
| SYNTH_NO_FLAT | A flag that disables flattening the hierarchy during synthesis, only flattening it after synthesis, mapping and optimizations. Enabled = 1, Disabled = 0 (Default: 0) |
| SYNTH_SHARE_RESOURCES | A flag that enables yosys to reduce the number of cells by determining shareable resources and merging them. Enabled = 1, Disabled = 0 (Default: 1) |
| SYNTH_ADDER_TYPE | Adder type to which the $add and $sub operators are mapped to. Possible values are YOSYS/FA/RCA/CSA; where YOSYS refers to using Yosys internal adder definition, FA refers to full-adder structure, RCA refers to ripple carry adder structure, and CSA refers to carry select adder. (Default: YOSYS) |
| LIB_SLOWEST | Points to the lib file, corresponding to the slowest corner, for max delay calculation during STA. (Default: $::env(PDK_ROOT)/$::env(PDK)/libs.ref/$::env(STD_CELL_LIBRARY)/lib/sky130_fd_sc_hd__ff_n40C_1v95.lib) |
| LIB_FASTEST | Points to the lib file, corresponding to the fastest corner, for min delay calculation during STA. (Default: $::env(PDK_ROOT)/$::env(PDK)/libs.ref/$::env(STD_CELL_LIBRARY)/lib/sky130_fd_sc_hd__ss_100C_1v60.lib) |
| LIB_TYPICAL | Library used for typical delay calculation during STA. (DefaultLIB_SYNTH) |
| CLOCK_BUFFER_FANOUT | Fanout of clock tree buffers. (Default: 16) |
| ROOT_CLK_BUFFER | Root clock buffer of the clock tree. (Default: sky130_fd_sc_hd__clkbuf_16) |
| CLK_BUFFER | Clock buffer used for inner nodes of the clock tree. (Default: sky130_fd_sc_hd__clkbuf_4) |
| CLK_BUFFER_INPUT | Input pin of the clock tree buffer. (Default: A) |
| CLK_BUFFER_OUTPUT | Output pin of the clock tree buffer. (Default: X) |
| BASE_SDC_FILE | Specifies the base sdc file to source before running Static Timing Analysis. (Default: $::env(OPENLANE_ROOT)/scripts/base.sdc) |
| VERILOG_INCLUDE_DIRS | Specifies the verilog includes directories. Optional. |
| SYNTH_FLAT_TOP | Specifies whether or not the top level should be flattened during elaboration. 1 = True, 0= False Default: 0. |
| IO_PCT | Specifies the percentage of the clock period used in the input/output delays. Ranges from 0 to 1.0. (Default: 0.2) |

### Floorplanning

| Variable | Description |
| --- | --- |
| FP_CORE_UTIL | The core utilization percentage. (Default: 50 percent) |
| FP_ASPECT_RATIO | The core's aspect ratio (height / width). (Default: 1) |
| FP_SIZING | Whether to use relative sizing by making use of FP_CORE_UTIL or absolute one using DIE_AREA. (Default: "relative" - accepts "absolute" as well) |
| DIE_AREA | Specific die area to be used in floorplanning. Specified as a 4-corner rectangle. Units in mm (Default: unset) |
| FP_IO_HMETAL | The metal layer on which to place the io pins horizontally (top and bottom of the die). (Default: 4) |
| FP_IO_VMETAL | The metal layer on which to place the io pins vertically (sides of the die) (Default: 3) |
| FP_IO_MODE | Decides the mode of the random IO placement option. 0=matching mode, 1=random equidistant mode (Default: 1) |
| FP_WELLTAP_CELL | The name of the welltap cell during welltap insertion. |
| FP_ENDCAP_CELL | The name of the endcap cell during endcap insertion. |
| FP_PDN_VOFFSET | The offset of the vertical power stripes on the metal layer 4 in the power distribution network (Default: 16.32) |
| FP_PDN_VPITCH | The pitch of the vertical power stripes on the metal layer 4 in the power distribution network (Default: 153.6) |
| FP_PDN_HOFFSET | The offset of the horizontal power stripes on the metal layer 5 in the power distribution network (Default: 16.65) |
| FP_PDN_HPITCH | The pitch of the horizontal power stripes on the metal layer 5 in the power distribution network (Default: 153.18) |
| FP_PDN_AUTO_ADJUST | Decides whether or not the flow should attempt to re-adjust the power grid, in order for it to fit inside the core area of the design, if needed. 1=enabled, 0 =disabled (Default: 1) |
| FP_TAPCELL_DIST | The horizontal distance between two tapcell columns (Default: 14) |
| FP_IO_VEXTEND | Extends the vertical io pins outside of the die by the specified units (Default: -1 Disabled) |
| FP_IO_HEXTEND | Extends the horizontal io pins outside of the die by the specified units (Default: -1 Disabled) |
| FP_IO_VLENGTH | The length of the vertical IOs in microns. (Default: 4) |
| FP_IO_HLENGTH | The length of the horizontal IOs in microns. (Default: 4) |
| FP_IO_VTHICKNESS_MULT | A multiplier for vertical pin thickness. Base thickness is the pins layer minwidth (Default: 2) |
| FP_IO_HTHICKNESS_MULT | A multiplier for horizontal pin thickness. Base thickness is the pins layer minwidth (Default: 2) |
| BOTTOM_MARGIN_MULT | The core margin, in multiples of site heights, from the bottom boundary. (Default: 4) |
| TOP_MARGIN_MULT | The core margin, in multiples of site heights, from the top boundary. (Default: 4) |
| LEFT_MARGIN_MULT | The core margin, in multiples of site widths, from the left boundary. (Default: 12) |
| RIGHT_MARGIN_MULT | The core margin, in multiples of site widths, from the right boundary. (Default: 12) |
| FP_PDN_CORE_RING | Enables adding a core ring around the design. More details on the control variables in the pdk configurations documentation. 0=Disable 1=Enable. (Default: 0) |
| FP_PDN_ENABLE_RAILS | Enables the creation of rails in the power grid. 0=Disable 1=Enable. (Default: 1) |
| FP_PDN_CHECK_NODES | Enables checking for unconnected nodes in the power grid. 0=Disable 1=Enable. (Default: 1) |
| FP_HORIZONTAL_HALO | Sets the horizontal halo around the tap and decap cells. The value provided is in microns. Default: 10 |
| FP_VERTICAL_HALO | Sets the vertical halo around the tap and decap cells. The value provided is in microns. Default: set to the value of FP_HORIZONTAL_HALO |
| DESIGN_IS_CORE | Controls the layers used in the power grid. Depending on whether the design is the core of the chip or a macro inside the core. 1=Is a Core, 0=Is a Macro (Default: 1) |
| FP_PIN_ORDER_CFG | Points to the pin order configuration file to set the pins in specific directions (S, W, E, N). Check this file as an example. If not set, then the IO pins will be placed based on one of the other methods depending on the rest of the configurations. (Default: NONE) |
| FP_CONTEXT_DEF | Points to the parent DEF file that includes this macro/design and uses this DEF file to determine the best locations for the pins. It must be used with FP_CONTEXT_LEF, otherwise it's considered non-existing. If not set, then the IO pins will be placed based on one of the other methods depending on the rest of the configurations. (Default: NONE) |
| FP_CONTEXT_LEF | Points to the parent LEF file that includes this macro/design and uses this LEF file to determine the best locations for the pins. It must be used with FP_CONTEXT_DEF, otherwise it's considered non-existing. If not set, then the IO pins will be placed based on one of the other methods depending on the rest of the configurations. (Default: NONE) |
| FP_DEF_TEMPLATE | Points to the DEF file to be used as a template when running apply_def_template. This will be used to exctract pin names, locations, shapes -excluding power and ground pins- as well as the die area and replicate all this information in the CURRENT_DEF. |
| VDD_NETS | Specifies the power nets/pins to be used when creating the power grid for the design. |
| GND_NETS | Specifies the ground nets/pins to be used when creating the power grid for the design. |
| SYNTH_USE_PG_PINS_DEFINES | Specifies the power guard used in the verilog source code to specify the power and ground pins. This is used to automatically extract VDD_NETS and GND_NET variables from the verilog, with the assumption that they will be order inout vdd1, inout gnd1, inout vdd2, inout gnd2, .... |

### Placement

| Variable | Description |
| --- | --- |
| PL_TARGET_DENSITY | The desired placement density of cells. It reflects how spread the cells would be on the core area. 1 = closely dense. 0 = widely spread (Default: 0.55) |
| PL_TIME_DRIVEN | Specifies whether the placer should use time driven placement. 0 = false, 1 = true (Default: 0) |
| PL_LIB | Specifies the library for time driven placement (Default: LIB_TYPICAL) |
| PL_BASIC_PLACEMENT | Specifies whether the placer should run basic placement or not (by running initial placement, increasing the minimum overflow to 0.9, and limiting the number of iterations to 20). 0 = false, 1 = true (Default: 0) |
| PL_SKIP_INITIAL_PLACEMENT | Specifies whether the placer should run initial placement or not. 0 = false, 1 = true (Default: 0) |
| PL_RANDOM_GLB_PLACEMENT | Specifies whether the placer should run random placement or not. This is useful if the design is tiny (less than 100 cells). 0 = false, 1 = true (Default: 0) |
| PL_RANDOM_INITIAL_PLACEMENT | Specifies whether the placer should run random placement or not followed by replace's initial placement. This is useful if the design is tiny (less than 100 cells). 0 = false, 1 = true (Default: 0) |
| PL_ROUTABILITY_DRIVEN | Specifies whether the placer should use routability driven placement. 0 = false, 1 = true (Default: 0) |
| PL_OPENPHYSYN_OPTIMIZATIONS | Specifies whether OpenPhySyn should be used to perform timing optimizations or not. 0 = false, 1 = true (Default: 0) |
| PSN_ENABLE_RESIZING | Enables driver resizing by OpenPhySyn. 0 = Disabled, 1 = Enabled (Default: 1) |
| PSN_ENABLE_PIN_SWAP | Enables pin swapping for timing optimization by OpenPhySyn. 0 = Disabled, 1 = Enabled (Default: 1) |
| PL_RESIZER_DESIGN_OPTIMIZATIONS | Specifies whether resizer design optimizations should be performed or not. 0 = false, 1 = true (Default: 1) |
| PL_RESIZER_TIMING_OPTIMIZATIONS | Specifies whether resizer timing optimizations should be performed or not. 0 = false, 1 = true (Default: 1) |
| PL_RESIZER_MAX_WIRE_LENGTH | Specifies the maximum wire length cap used by resizer to insert buffers. If set to 0, no buffers will be inserted. Value in microns. (Default: 0) |
| LIB_OPT | Points to the lib file, corresponding to the slowest corner, for max delay calculation during OpenPhySyn optimizations. This is usually a trimmed version of LIB_SLOWEST. Default: $::env(TMP_DIR)/opt.lib |
| LIB_RESIZER_OPT | Points to the lib file, corresponding to the slowest corner, for max delay calculation during resizer optimizations. This is copy of LIB_SLOWEST. Default: $::env(TMP_DIR)/resizer.lib |
| DONT_USE_CELLS | The list of cells to not use during resizer optimizations. Default: the contents of DRC_EXCLUDE_CELL_LIST. |
| PL_ESTIMATE_PARASITICS | Specifies whether or not to run STA after global placement using OpenROAD's estimate_parasitics -placement and generates reports under logs/placement. 1 = Enabled, 0 = Disabled. (Default: 1) |
| PL_DIAMOND_SEARCH_HEIGHT | Specifies the diamond search height used for legalizing the cells during detailed placement. The search width is calculated internally as heigh*5. For designs that contain big macros, increasing this value to above 400 will allow for more search space and more potentail for successful legalization. (Default: 100) |
| PL_OPTIMIZE_MIRRORING | Specifies whether or not to run an optimize_mirroring pass whenever detailed placement happens. This pass will mirror the cells whenever possible to optimize the design. 1 = Enabled, 0 = Disabled. (Default: 1) |
| PL_RESIZER_BUFFER_INPUT_PORTS | Specifies whether or not to insert buffers on input ports whenever resizer optimizations are run. For this to be used, PL_RESIZER_DESIGN_OPTIMIZATIONS must be set to 1. 1 = Enabled, 0 = Disabled. (Default: 1) |
| PL_RESIZER_BUFFER_OUTPUT_PORTS | Specifies whether or not to insert buffers on output ports whenever resizer optimizations are run. For this to be used, PL_RESIZER_DESIGN_OPTIMIZATIONS must be set to 1. 1 = Enabled, 0 = Disabled. (Default: 1) |

### CTS

| Variable | Description |
| --- | --- |
| CTS_TARGET_SKEW | The target clock skew in picoseconds. (Default: 200 ps) |
| CTS_ROOT_BUFFER | The name of cell inserted at the root of the clock tree. |
| CLOCK_TREE_SYNTH | Enable clock tree synthesis for tirtonCTS. (Default: 1) |
| CTS_TOLERANCE | An integer value that represents a tradeoff of QoR and runtime. Higher values will produce smaller runtime but worse QoR (Default: 100) |
| CTS_SINK_CLUSTERING_SIZE | Specifies the maximum number of sinks per cluster. (Default: 20) |
| CTS_SINK_CLUSTERING_MAX_DIAMETER | Specifies maximum diameter (in micron) of sink cluster. (Default: 50) |
| CTS_REPORT_TIMING | Specifies whether or not to run STA after clock tree synthesis using OpenROAD's estimate_parasitics -placement and generates reports under logs/cts. 1 = Enabled, 0 = Disabled. (Default: 1) |
| LIB_CTS | The liberty file used for CTS. By default, this is the LIB_SYNTH_COMPLETE minus the cells with drc errors as specified by the drc exclude list. (Default: $::env(TMP_DIR)/cts.lib) |

### Routing

| Variable | Description |
| --- | --- |
| GLB_RT_MINLAYER | The number of lowest layer to be used in routing. (Default: 1) |
| GLB_RT_MAXLAYER | The number of highest layer to be used in routing. (Default: 6) |
| GLB_RT_ADJUSTMENT | Reduction in the routing capacity of the edges between the cells in the global routing graph. Values range from 0 to 1. 1 = most reduction, 0 = least reduction (Default: 0) |
| GLB_RT_L1_ADJUSTMENT | Reduction in the routing capacity of the edges between the cells in the global routing graph but specific to li1 layer in sky130A. Values range from 0 to 1 (Default: 0.99) |
| GLB_RT_L2_ADJUSTMENT | Reduction in the routing capacity of the edges between the cells in the global routing graph but specific to met1 in sky130A. Values range from 0 to 1 (Default: 0) |
| GLB_RT_L3_ADJUSTMENT | Reduction in the routing capacity of the edges between the cells in the global routing graph but specific to met2 in sky130A. Values range from 0 to 1 (Default: 0) |
| GLB_RT_L4_ADJUSTMENT | Reduction in the routing capacity of the edges between the cells in the global routing graph but specific to met3 in sky130A. Values range from 0 to 1 (Default: 0) |
| GLB_RT_L5_ADJUSTMENT | Reduction in the routing capacity of the edges between the cells in the global routing graph but specific to met4 in sky130A. Values range from 0 to 1 (Default: 0) |
| GLB_RT_L6_ADJUSTMENT | Reduction in the routing capacity of the edges between the cells in the global routing graph but specific to met5 in sky130A. Values range from 0 to 1 (Default: 0) |
| GLB_RT_UNIDIRECTIONAL | Allow unidirectional routing. 0 = false, 1 = true (Default: 1) |
| GLB_RT_ALLOW_CONGESTION | Allow congestion in the resultign guides. 0 = false, 1 = true (Default: 0) |
| GLB_RT_OVERFLOW_ITERS | The maximum number of iterations waiting for the overflow to reach the desired value. (Default: 50) |
| GLB_RT_TILES | The size of the GCELL used by Fastroute during global routing. (Default: 15) |
| GLB_RT_ESTIMATE_PARASITICS | Specifies whether or not to run STA after global routing using OpenROAD's estimate_parasitics -global_routing and generates reports under logs/routing. 1 = Enabled, 0 = Disabled. (Default: 1) |
| ROUTING_CORES | Specifies the number of threads to be used in TritonRoute. (Default: 4) |
| GLB_RT_MAX_DIODE_INS_ITERS | Controls the maximum number of iterations at which re-running Fastroute for diode insertion stops. Each iteration ARC detects the violations and FastRoute fixes them by inserting diodes, then producing the new DEF. The number of antenna violations is compared with the previous iteration and if they are equal or the number is greater the iterations stop and the DEF from the previous iteration is used in the rest of the flow. If the current antenna violations reach zero, the current def will be used and the iterations will not continue. This option is only available in DIODE_INSERTION_STRATEGY = 3. (Default: 1) |
| GLB_RT_OBS | Specifies custom obstruction to be added prior to global routing. Comma separated list of layer and coordinates: layer llx lly urx ury. (Example: li1 0 100 1000 300, met5 0 0 1000 500) (Default: unset) |
| ROUTING_OPT_ITERS | Specifies the maximum number of optimization iterations during Detailed Routing in TritonRoute. (Default: 64) |
| GLOBAL_ROUTER | Specifies which global router to use. Values: fastroute or cugr. (Default: fastroute) |
| DETAILED_ROUTER | Specifies which detailed router to use. Values: tritonroute, tritonroute_or, or drcu. (Default: tritonroute) |

### Magic

| Variable | Description |
| --- | --- |
| MAGIC_PAD | A flag to pad the views generated by magic (.mag, .lef, .gds) with one site. 1 = Enabled, 0 = Disabled (Default: 0 ) |
| MAGIC_ZEROIZE_ORIGIN | A flag to move the layout such that it's origin in the lef generated by magic is 0,0. 1 = Enabled, 0 = Disabled (Default: 1 ) |
| MAGIC_GENERATE_GDS | A flag to generate gds view via magic . 1 = Enabled, 0 = Disabled (Default: 1 ) |
| MAGIC_GENERATE_LEF | A flag to generate lef view via magic . 1 = Enabled, 0 = Disabled (Default: 1 ) |
| MAGIC_GENERATE_MAGLEF | A flag to generate maglef view via magic . 1 = Enabled, 0 = Disabled (Default: 1 ) |
| MAGIC_WRITE_FULL_LEF | A flag to specify whether or not the output LEF should include all shapes inside the macro or an abstracted view of the macro lef view via magic . 1 = Full View, 0 = Abstracted View (Default: 0 ) |
| MAGIC_DRC_USE_GDS | A flag to choose whether to run the magic DRC checks on GDS or not. If not, then the checks will be done on the DEF/LEF. 1 = GDS, 0 = DEF/LEF (Default: 1 ) |
| MAGIC_EXT_USE_GDS | A flag to choose whether to run the magic extractions on GDS or DEF/LEF. If GDS was used Device Level LVS will be run. Otherwise, blackbox LVS will be run. 1 = GDS, 0 = DEF/LEF (Default: 0 ) |
| MAGIC_INCLUDE_GDS_POINTERS | A flag to choose whether to include GDS pointers in the generated mag files or not. 1 = Enabled, 0 = Disabled (Default: 0 ) |
| MAGIC_DISABLE_HIER_GDS | A flag to disable cif hier and array during GDS-II writing.* 1=Enabled <so this hier gds will be disabled>, 0=Disabled <so this hier gds will be enabled>. (Default: 1 ) |

- Tim Edwards's Explanation on disabling hier gds: The following is an explanation by Tim Edwards, provided in a slack thread, on how this affects the GDS writing process: "Magic can take a very long time writing out GDS while checking hierarchical interactions in a standard cell layout. If your design is all digital, I recommend using "gds *hier write disable" before "gds write" so that it does not try to resolve hierarchical interactions (since by definition, standard cells are designed to just sit next to each other without creating DRC issues).  That can actually make the difference between a 20 hour GDS write and a 2 minute GDS write.  For a standard cell design that takes up the majority of the user space, a > 24 hour write time (without disabling the hierarchy checks) would not surprise me."

### LVS

| Variable | Description |
| --- | --- |
| LVS_INSERT_POWER_PINS | Enables power pins insertion before running lvs. 1 = Enabled, 0 = Disabled (Default: 1 ) |
| LVS_CONNECT_BY_LABEL | Enables connections by label in LVS by skipping extract unique in magic extractions. Default: 0 |
| YOSYS_REWRITE_VERILOG | Enables yosys to rewrite the verilog before LVS producing a canonical verilog netlist with verbose wire declarations. This flag will be ignored if LEC_ENABLE is 1, and it will be rewritten anyways. 1 = Enabled, 0 = Disabled (Default: 0 ) |

### Misc

| Variable | Description |
| --- | --- |
| PDK | Specifies the process design kit (PDK). (Default: sky130A ) |
| STD_CELL_LIBRARY | Specifies the standard cell library to be used under the specified PDK. (Default: sky130_fd_sc_hd ) |
| PDK_ROOT | Specifies the folder path of the PDK. It searches for a config.tcl in $PDK_ROOT/$PDK/libs.tech/openlane/ directory and at least have one standard cell library config defined in $PDK_ROOT/$PDK/libs.tech/openlane/$STD_CELL_LIBRARY. |
| CELL_PAD | Cell padding; increases the width of cells. (Default: 4 microns -- 4 sites) |
| DIODE_PADDING | Diode cell padding; increases the width of diode cells during placement checks. (Default: 2 microns -- 2 sites) |
| WIRE_RC_LAYER | The metal layer used in estimate parastics set_wire_rc. Should be moved to PDK configurations later.. Default: met1. |
| MERGED_LEF_UNPADDED | Points to merged_unpadded.lef by default. it contains the technology LEF for the used STD_CELL_LIBRARY merged with the LEF file for all the cells. |
| MERGED_LEF | points to merged.lef, which is merged_unpadded.lef but with cell padding. This is controlled by CELL_PAD. |
| NO_SYNTH_CELL_LIST | Specifies the file that contains the don't-use-cell-list to be excluded from the liberty file during synthesis. If it's not defined, this path is searched $::env(PDK_ROOT)/$::env(PDK)/libs.tech/openlane/$::env(STD_CELL_LIBRARY)/no_synth.cells and if it's not found, then the original liberty will be used as is. |
| DRC_EXCLUDE_CELL_LIST | Specifies the file that contains the don't-use-cell-list to be excluded from the liberty file during synthesis and timing optimizations. If it's not defined, this path is searched $::env(PDK_ROOT)/$::env(PDK)/libs.tech/openlane/$::env(STD_CELL_LIBRARY)/drc_exclude.cells and if it's not found, then the original liberty will be used as is. In other words, DRC_EXCLUDE_CELL_LIST contain the only excluded cell list in timing optimizations. |

### Flow control

| Variable | Description |
| --- | --- |
| USE_GPIO_PADS | Decides whether or not to use the gpio pads in routing by merging their LEF file set in ::env(USE_GPIO_ROUTING_LEF) and blackboxing their verilog modules set in ::env(GPIO_PADS_VERILOG). 1=Enabled, 0=Disabled. (Default: 0) |
| LEC_ENABLE | Enables logic verification using yosys, for comparing each netlist at each stage of the flow with the previous netlist and verifying that they are logically equivalent. Warning: this will increase the runtime significantly. 1 = Enabled, 0 = Disabled (Default: 0) |
| RUN_ROUTING_DETAILED | Enables detailed routing. 1 = Enabled, 0 = Disabled (Default: 1) |
| RUN_MAGIC | Enables running magic and GDSII streaming. 1 = Enabled, 0 = Disabled (Default: 1) |
| RUN_KLAYOUT | Enables running Klayout and GDSII streaming. 1 = Enabled, 0 = Disabled (Default: 1) |
| RUN_KLAYOUT_DRC | Enables running Klayout DRC on GDS-II produced by magic. 1 = Enabled, 0 = Disabled (Default: 1) |
| KLAYOUT_DRC_KLAYOUT_GDS | Enables running Klayout DRC on GDS-II produced by Klayout. 1 = Enabled, 0 = Disabled (Default: 0) |
| RUN_KLAYOUT_XOR | Enables running Klayout XOR on 2 GDS-IIs, the defaults are the one produced by magic vs the one produced by klayout. 1 = Enabled, 0 = Disabled (Default: 1) |
| KLAYOUT_XOR_GDS | If RUN_KLAYOUT_XOR is enabled, this will enable producing a GDS output from the XOR along with it's PNG export. 1 = Enabled, 0 = Disabled (Default: 1) |
| KLAYOUT_XOR_XML | If RUN_KLAYOUT_XOR is enabled, this will enable producing an XML output from the XOR. 1 = Enabled, 0 = Disabled (Default: 1) |
| TAKE_LAYOUT_SCROT | Enables running Klayout to take a PNG screenshot of the produced layout (currently configured to run on the results of each stage).1 = Enabled, 0 = Disabled (Default: 1) |
| RUN_SIMPLE_CTS | Enables inserting simple clock tree after synthesis .1 = Enabled, 0 = Disabled (Default: 0) |
| FILL_INSERTION | Enables fill cells insertion after cts (if enabled) .1 = Enabled, 0 = Disabled (Default: 1) |
| TAP_DECAP_INSERTION | Enables tap and decap cells insertion after floorplanning (if enabled) .1 = Enabled, 0 = Disabled (Default: 1) |
| DIODE_INSERTION_STRATEGY | Specifies the insertion strategy of diodes to be used in the flow. 0 = No diode insertion, 1 = Spray diodes, 2 = insert fake diodes and replace them with real diodes if needed. 3= use FastRoute Antenna Avoidance flow, 4 = Use Sylvian's Custom Script for diode insertion on design pins and smartly inserting needed diodes inside the design, 5 = a mix of strategy 2 and 4. (Default: 3) |
| WIDEN_SITE | Specifies the new virtual width of the site to be used in all stages up to diode insertion, then switched back to the original site width. It can be either a factor or an absolute value controlled by WIDEN_SITE_IS_FACTOR (Default: 1) |
| WIDEN_SITE_IS_FACTOR | Specifies whether the given WIDEN_SITE should be treated as a factor or an absolute value. 0 = absolute, 1 = factor (Default: 1) |
| USE_ARC_ANTENNA_CHECK | Specifies whether to use the openroad ARC antenna checker or magic antenna checker. 0=magic antenna checker, 1=ARC OR antenna checker (Default: 1) |
| RUN_SPEF_EXTRACTION | Specifies whether or not to run SPEF extraction on the routed DEF. 1=enabled 0=disabled Default: 1 |
| SPEF_WIRE_MODEL | Specifies the wire model used in SPEF extraction. Options are L or Pi Default: L |
| SPEF_EDGE_CAP_FACTOR | Specifies the edge capacitance factor used in SPEF extraction. Ranges from 0 to 1 Default: 1 |
| GENERATE_FINAL_SUMMARY_REPORT | Specifies whether or not to generate a final summary report after the run is completed. Check command generate_final_summary_report. 1=enabled 0=disabled Default: 1 |
| MAGIC_CONVERT_DRC_TO_RDB | Specifies whether or not generate a Calibre RDB out of the magic.drc report. Result is saved in <run_path>/results/magic/. 1=enabled 0=disabled Default: 1 |
| RUN_CVC | Runs CVC on the output spice, which is a Circuit Validity Checker. Voltage aware ERC checker for CDL netlists. Thus, it controls the command run_lef_cvc. 1=Enabled, 0=Disabled. Default: 1 |

### Checkers

| Variable | Description |
| --- | --- |
| CHECK_UNMAPPED_CELLS | Checks if there are unmapped cells after synthesis and aborts if any was found. 1 = Enabled, 0 = Disabled (Default: 1) |
| CHECK_ASSIGN_STATEMENTS | Checks for assign statement in the generated gate level netlist and aborts of any was found.1 = Enabled, 0 = Disabled (Default: 0) |
| QUIT_ON_TR_DRC | Checks for DRC violations after routing and exits the flow if any was found. 1 = Enabled, 0 = Disabled (Default: 1) |
| QUIT_ON_MAGIC_DRC | Checks for DRC violations after magic DRC is executed and exits the flow if any was found. 1 = Enabled, 0 = Disabled (Default: 1) |
| QUIT_ON_ILLEGAL_OVERLAPS | Checks for illegal overlaps during magic extraction. In some cases, these imply existing undetected shorts in the design. It also exits the flow if any was found. 1 = Enabled, 0 = Disabled (Default: 1) |
| QUIT_ON_LVS_ERROR | Checks for LVS errors after netgen LVS is executed and exits the flow if any was found. 1 = Enabled, 0 = Disabled (Default: 1) |