DISCLAIMER: The following sections are to give you an understanding of what happens under the hood in the Makefile.

# Prerequisites

- Magic VLSI Layout Tool is needed to run open_pdks -- version >= 8.3.60*

# Setting up the PDK: skywater-pdk

- Clone and build at least one skywater-pdk standard cell Library inside the pdks directory:

  - To setup one standard cell library only

export PDK_ROOT=<absolute path to where skywater-pdk and open_pdks will reside>

cd  $PDK_ROOT

git clone https://github.com/google/skywater-pdk.git

cd skywater-pdk

git checkout db2e06709dc3d876aa6b74a5f3893fa5f1bc2a6e

git submodule update --init libraries/sky130_fd_sc_hd/latest

git submodule update --init libraries/sky130_fd_sc_hvl/latest

git submodule update --init libraries/sky130_fd_io/latest

make timing

  - To setup other SCLs:

    - replace sky130_fd_sc_hd with any of the following list:

      - sky130_fd_sc_hs

      - sky130_fd_sc_ms

      - sky130_fd_sc_ls

      - sky130_fd_sc_hdll

- Setup the configurations and tech files for Magic, Netgen, OpenLANE using open_pdks:

cd $PDK_ROOT

git clone git://opencircuitdesign.com/open_pdks

cd open_pdks

git checkout b9ffc1fd1cfc26cbca85a61c287ac799721f6e6a

./configure --enable-sky130-pdk=$PDK_ROOT/skywater-pdk/libraries --with-sky130-local-path=$PDK_ROOT --enable-sram-sky130=disabled

cd sky130

make

make install-local

Note: You can use different directories for sky130-source and local-path. However, in the instructions we are using $PDK_ROOT to facilitate the installation process

WARNING: Please, don't move sk130A from the installed directory because the generated .mag files contain absolute paths. Moving it will result in producing an invalid GDS.

- To set the STD_CELL_LIBRARY (the default value is set to sky130_fd_sc_hd)

  - Open configuration/general.tcl

  - set STD_CELL_LIBRARY to one of the following:

- sky130_fd_sc_hs

- sky130_fd_sc_ms

- sky130_fd_sc_ls

- sky130_fd_sc_hdll

Refer to this for more details on the structure.