## Interactive Mode

You may run the flow interactively by using the -interactive option:

./flow.tcl -interactive

A tcl shell will be opened where the openlane package is automatically sourced:

% package require openlane

Then, you should be able to run the following main commands:

- Any tcl command.

- prep -design <design> -tag <tag> -config <config> -init_design_config -overwrite similar to the command line arguments, design is required and the rest is optional

- run_synthesis

- run_floorplan

- run_placement

- run_cts

- run_routing

- write_powered_verilog followed by set_netlist $::env(lvs_result_file_tag).powered.v

- run_magic

- run_magic_spice_export

- run_magic_drc

- run_lvs

- run_antenna_check

The above commands can also be written in a file and passed to flow.tcl:

./flow.tcl -interactive -file <file>

A more detailed list of all the commands supported by openlane could be found here.

Note 1: Currently, configuration variables have higher priority over the above commands so if RUN_MAGIC is 0, command run_magic will have no effect.

Note 2: Currently, most of these commands must be run in the flow sequence and no steps should be skipped.

Note 3: You can pass the -design, -tag, etc.. flags to ./flow.tcl -interactive directly without the need of entering the interactive mode and then executing the prep command.