|  | design | config | total_runtime | DIEAREA_mm^2 | OpenDP_Util | cell_count | tritonRoute_violations | Short_violations | Magic_violations | antenna_violations | wns | CLOCK_PERIOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | digital_pll_sky130_fd_sc_hd | config_TEST_SW_HS | 0h0m36s | -1e-06 | -1 | -1 | -1 | -1 | -1 | -1 | -1.0 | 15.8 |
| 1 | inverter | config_TEST_SW_HS | 0h5m55s | 0.001836 | 4 | 1 | 0 | 0 | 0 | 0 | 0.0 | 10.0 |
| 2 | manual_macro_placement_test | config_TEST_SW_HS | 0h6m56s | 0.102024 | 1 | 2 | 0 | 0 | 0 | 2 | 0.0 | 100.0 |
| 3 | zipdiv | config_HS_first_rerun | 0h7m29s | 0.048563999999999996 | 44 | 1080 | 0 | 0 | 0 | 1 | 0.0 | 19.09 |
| 4 | xtea | config_HS_first_rerun | 0h8m41s | 0.098226 | 45 | 2418 | 0 | 0 | 0 | 6 | 0.0 | 26.03 |
| 5 | s44 | config_TEST_SW_HS | 0h13m26s | 0.043416 | 7 | 97 | 0 | 0 | 0 | 0 | 0.0 | 30.0 |
| 6 | spm | config_TEST_SW_HS | 0h16m33s | 0.01375 | 54 | 287 | 0 | 0 | 0 | 0 | 0.0 | 10.0 |
| 7 | cic_decimator | config_TEST_SW_HS | 0h18m30s | 0.031806 | 48 | 801 | 0 | 0 | 0 | 0 | 0.0 | 10.0 |
| 8 | usb | config_TEST_SW_HS | 0h19m58s | 0.041356 | 46 | 837 | 0 | 0 | 0 | 0 | 0.0 | 12.55 |
| 9 | usb_cdc_core | config_TEST_SW_HS | 0h36m47s | 0.072574 | 51 | 2079 | 0 | 0 | 0 | 1 | 0.0 | 15.6 |
| 10 | APU | config_TEST_SW_HS | 0h41m20s | 0.137956 | 44 | 3095 | 0 | 0 | 0 | 1 | 0.0 | 17.0 |
| 11 | wbqspiflash | config_TEST_SW_HS | 0h53m51s | 0.12209400000000001 | 41 | 2553 | 0 | 0 | 0 | 2 | 0.0 | 18.86 |
| 12 | synth_ram | config_TEST_SW_HS | 1h11m43s | 0.2835 | 56 | 6518 | 0 | 0 | 0 | 3 | 0.0 | 50.0 |
| 13 | point_scalar_mult | config_HS_first_rerun | 3h13m43s | 3.6461339999999995 | 27 | 55243 | 0 | 0 | 946 | 75 | -21.64 | 10.0 |
| 14 | y_dct | config_HS_first_rerun | 5h41m7s | 8.16525 | 20 | 78291 | 0 | 0 | 48 | 113 | 0.0 | 20.52 |
| 15 | ocs_blitter | config_TEST_SW_HS | 1h28m5s | 0.304096 | 49 | 6996 | 0 | 0 | 0 | 6 | -18.79 | 18.2 |
| 16 | md5 | config_TEST_SW_HS | 1h36m27s | 0.268786 | 47 | 5720 | 0 | 0 | 0 | 6 | 0.0 | 37.05 |
| 17 | ldpc_decoder_802_3an | config_TEST_SW_HS | 1h52m10s | -1e-06 | 18 | 92279 | -1 | -1 | -1 | -1 | -24.93 | 77.0 |
| 18 | 151 | config_HS_first_rerun | 6h18m4s | 7.120836 | 27 | 73184 | 0 | 0 | 11 | 120 | -179.09 | 50.0 |
| 19 | PPU | config_TEST_SW_HS | 2h7m20s | 0.880726 | 28 | 9398 | 0 | 0 | 0 | 10 | -10.16 | 18.0 |
| 20 | genericfir | config_TEST_SW_HS | 2h41m41s | 0.42700600000000005 | 50 | 9475 | 0 | 0 | 0 | 3 | 0.0 | 10.0 |
| 21 | BM64 | config_TEST_SW_HS | 2h56m31s | 0.706384 | 27 | 8475 | 0 | 0 | 0 | 19 | 0.0 | 20.0 |
| 22 | y_huff | config_TEST_SW_HS | 3h11m0s | 0.58135 | 40 | 10614 | 0 | 0 | 4 | 13 | -1.08 | 11.35 |
| 23 | des | config_TEST_SW_HS | 3h12m14s | 0.618526 | 40 | 15968 | 0 | 0 | 2 | 21 | 0.0 | 20.0 |
| 24 | picorv32a | config_TEST_SW_HS | 3h18m57s | 0.5828760000000001 | 47 | 14452 | 0 | 0 | 0 | 8 | 0.0 | 24.73 |
| 25 | chacha | config_TEST_SW_HS | 3h33m32s | 0.899596 | 36 | 16517 | 0 | 0 | 0 | 11 | -8.85 | 26.01 |
| 26 | usbf_device | config_TEST_SW_HS | 3h53m19s | 0.609897 | 48 | 15403 | 0 | 0 | 0 | 5 | 0.0 | 14.6 |
| 27 | ldpcenc | config_TEST_SW_HS | 3h53m37s | 1.07635 | 38 | 17597 | 0 | 0 | 0 | 64 | -33.26 | 12.9 |
| 28 | sha3 | config_TEST_SW_HS | 3h56m13s | 0.628785 | 45 | 14341 | 0 | 0 | 0 | 13 | -2.06 | 12.71 |
| 29 | blabla | config_TEST_SW_HS | 4h5m58s | 1.268946 | 22 | 12838 | 0 | 0 | 0 | 41 | 0.0 | 65.0 |
| 30 | aes_cipher | config_TEST_SW_HS | 4h11m32s | 0.66825 | 36 | 14636 | 0 | 0 | 75 | 6 | 0.0 | 10.09 |
| 31 | aes | config_TEST_SW_HS | 4h29m28s | 1.487124 | 29 | 18853 | 0 | 0 | 0 | 27 | -0.16 | 25.9 |
| 32 | aes_core | config_TEST_SW_HS | 4h31m51s | 1.101394 | 38 | 18368 | 0 | 0 | 0 | 32 | 0.0 | 21.28 |
| 33 | sha512 | config_TEST_SW_HS | 4h33m45s | 1.511614 | 37 | 23942 | 0 | 0 | 0 | 37 | -33.84 | 39.27 |
| 34 | salsa20 | config_TEST_SW_HS | 4h34m47s | 1.478592 | 29 | 19609 | 0 | 0 | 2 | 41 | 0.0 | 36.73 |
| 35 | aes128 | config_TEST_SW_HS | 4h54m45s | 2.431984 | 30 | 41497 | 0 | 0 | 317 | 52 | 0.0 | 11.43 |
| 36 | aes192 | config_TEST_SW_HS | 5h38m19s | 3.5815 | 25 | 46511 | 0 | 0 | 720 | 72 | 0.0 | 12.37 |
| 37 | aes256 | config_TEST_SW_HS | 5h54m50s | 3.113404 | 36 | 60227 | 0 | 0 | 657 | 93 | 0.0 | 15.0 |
| 38 | des3 | config_TEST_SW_HS | 6h14m46s | 3.5588260000000003 | 24 | 50296 | 0 | 0 | 1012 | 65 | -12.35 | 7.59 |
| 39 | point_add | config_TEST_SW_HS | 6h18m24s | 2.613016 | 34 | 50560 | 0 | 0 | 1576 | 56 | -21.64 | 10.0 |
| 40 | jpeg_encoder | config_TEST_SW_HS | 7h11m39s | 3.7888059999999997 | 26 | 58308 | 0 | 0 | 15 | 88 | -14.11 | 15.78 |
| 41 | sound | config_TEST_SW_HS | 23h43m18s | 11.387194000000001 | 34 | 200535 | 0 | 0 | 9 | 136 | 0.0 | 136.83 |